DOC. NO. _______

PROJECT NAME

PROJECT ADDRESS

NOTES TO DESIGN ENGINEER:

Blue information = notes for you, which you can delete before printing.

Red blanks are for you to fill in with the appropriate information.

COMPREHENSIVE TECHNICAL SPECIFICATIONS

SECTION (     ) STELCOR– DRILLED-IN DISPLACEMENT MICROPILES (DDM)

PART 1 - GENERAL

1.01	SUMMARY

A.	Section Includes

1.	DDM consists of one soil displacement head and continuous reverse flighted auger attached to a central steel shaft.  Extend piles by adding shaft extensions.

2.	DDM piles and related hardware by Ideal Manufacturing, Inc.

999 Picture Parkway, Webster, NY 14580      1-800-789-4810

B.	Related Sections

1.	Section _______ - Earthwork

2.	Section _______ - Structural Concrete

1.02	REFERENCES

A.	Conform to applicable requirements of the Building Code of _____________and applicable requirements of other	referenced documents.

B.	References include documents from:

1.	ASTM - American Society for Testing and Materials

a.	ASTM A36/A 36M - "Structural Steel"

b.	ASTM A29/A 29M - "Steel Bars, Carbon and Alloy, Hot-Wrought and Cold Finished"

c.	ASTM A53 - "Pipe, Steel, Black and Hot-Dipped, Zinc-Coated Welded and Seamless"

d.	ASTM A153 - "Zinc Coating (Hot-Dip) on Iron and Steel Hardware"

e.	ASTM 572 - "Latest Revision, HSLA Columbian-Vanadium Steels of Structural Quality"

f.	ASTM A607 - "Steel, Shaft and Strip, High-Strength, Low-Alloy Chromium or Vanadium,					or Both, Hot-Rolled and Cold-Rolled"

g.	ASTM SAE J429 - "Mechanical and Material Requirements for Externally Threaded Fasteners"

2.	ACI - American Concrete Institute

a.	ACI 301 - "Specifications for Structural Concrete for Buildings"

3.	PTI - Post Tensioning Institute

4.	API - American Petroleum Institute

_______________________________________________________________________________________________

1.03	SYSTEM DESCRIPTION

A.	Furnish all labor, materials, equipment and services for the design (including design submittals) and installation	of all DDM, in accordance with Drawings and Specification, including cut-offs and Load Transfer Device

Installation.

B.	Design DDM system to support loads as indicated on Drawings and outlined in this Section.  Submit DDM design calculations and other pertinent data for approval as specified in Submittals below.

1.	Obtain Architect's approval of design calculations and drawings before commencing pile installation.

Approval of submittals does not relieve Contractor of responsibility for performing the pile installation 			in accordance with Contract Documents.

1.04	SUBMITTALS

A.	Comply with requirements of Section ________

B.	Pre-Installation Submittals:  Submit the following items for approval not less than 14 days prior to commencing

pile installation.

1.	Delegated Design Data - Submit the following data, sealed by Professional Engineer registered in the State of __________:

a.	Calculations for pile design capacities.

b.	Shop drawings detailing grout column diameters, steel core diameter, wall thickness, yield, tensile                           and grade, displacement head configuration, length, and reverse auger flighting.

c.	Details of installation sequence and equipment to be used in pile installation.

d.	Sample copies of daily pile reports/field reports to be used.

C.	Construction Submittals:  Submit the following items on regular and timely basis:

1.	Record of daily pile installation

D.	Post-Installation Submittals:  Submit the following items upon completion of pile installation:

1.	Actual pile location drawing as specified in Part 3 - Field Quality Control.

E.	Quality Control Submittals

1.	Qualifications Certification:  Submit written certification or similar documentation signed by applicable

subcontractor, Prime Contractor and manufacturer (where applicable) indicating compliance with

applicable "Qualifications" requirements specified below in "Quality Assurance" section.

2.	Installer Experience Listing:  Submit list of completed projects using similar products to those proposed

for this project, including owner's contact and telephone number for each project, demonstrating compliance with applicable "Qualifications" requirements specified in "Quality Assurance" article.

______________________________________________________________________________________________

1.05	QUALITY ASSURANCE

A.	Qualifications

1.	Installer:  Certified Installer (Certified by DDM Manufacturer) able to demonstrate sufficient competent personnel to complete specified construction.  Capable of providing Certified job superintendent or foreman and ensuring such supervision will be present at Site during pile construction.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A.	For convenience, details and specifications have been based on the following specified product/manufacturer:

[See DDM Design Guidelines to fill this section.

Example:  Ideal Foundation Systems Product ‘SC1200-55036180’, as Manufactured by Ideal Manufacturing, Inc. 1-800-789-4810.

_________________________________________________________________________________________

_________________________________________________________________________________________

_________________________________________________________________________________________

_________________________________________________________________________________________

_________________________________________________________________________________________

_________________________________________________________________________________________

_________________________________________________________________________________________

2.02	MATERIALS

A.	Soil Displacement Head:

1.	Cold Rolled ASTM A572 Gr.50

B.	Pile/Anchor Shaft:

[See table on page 2 of DDM Design Guidelines to fill this section.

Example:  5.50” O.D. x 0.361” W.T., 80 ksi min. yield – New Secondary Steel Pipe]

C.	Steel Pile Cap:

1.	Plate ASTM A572 Gr.50

2.	Sleeve: 55 ksi min. yield.

D.	Grout:

- Grout to exceed 4,000 PSI at 28 days.

____________________________________________________________________________________________

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Verification of Conditions:  Examine conditions under which piles are to be installed in coordination with Installer

of materials and components specified in this Section and notify affected Prime Contractors and Architect in

writing of any conditions detrimental to proper and timely installation.  Do not proceed with installation until

unsatisfactory conditions have been corrected in a manner acceptable to Installer.

1.	When Installer confirms conditions as acceptable to ensure proper and timely installation and to

ensure requirements for applicable warranty or guarantee can be satisfied, submit to Architect written

confirmation from applicable Installer.  Failure to submit written confirmation and subsequent installation

will be assumed to indicate conditions are acceptable to Installer.

Optional - May be another party responsible for this item

3.02	PREPARATION

A.	Employ licensed land surveyor or registered professional engineer to establish all lines and grades required

for pile installation.

3.03	INSTALLATION

A.	Pile Installation:

1.	Provide installation equipment capable of installing pile of required minimum diameter to design depth.

2.	Position DDM pile in accordance with the contract documents.

3.	Use only manufacturer-approved connectors, adapter and accessories.

4.	All welding to be in accordance with AWS D1.1. Welders must be certified in accordance with AWS.

5.	Monitor and record depth of pile penetration. Provide torque monitoring device as part of the installing

unit.  Monitor and record torque applied during the installation of each pile at specific depths. Torque

monitoring is purely informational as a resource to engineering and installation contractor.  All design

capacities are derived from engineering calculations and verified by load testing.

6.	Remove encountered obstructions, or relocate DDM as required.  Relocation of DDM must be

approved by A/E.  Obstructions and relocations shall be considered "extra work."

7.        Provide high shear grout mixer and pump with a minimum three cubic feet per minute.

8.	Provide record book for all pile installation information.

3.04	FIELD QUALITY CONTROL		may be the same contractor as item 3.02

A.	Survey of Piles (Record Drawings) - Prime Contractor

1.	Testing and survey work required to establish pile locations and elevations.  Record drawings and

other ancillary operations required for completion of pile installation.

2.	Accurately locate each pile by means of survey performed by licensed surveyor or registered professional

engineer.  Record survey data with other required information on reproducible drawing.

______________________________________________________________________________________________

3.	Include the following information on record drawings:

a.	Each pile identified by separate number

b.	Angle of pile installation

c.	Elevation of each pile top

d.	Plan location of each pile

e.	Deviation from plan location in inches, measured to nearest 1/4 inch

f.	Grout take and torque reading for entire pile installation

g.	Description of lead section and extensions installed

h.	Soil Displacement Head size and shaft size

4.	Furnish prints of record drawings to Architect at completion of pile installation.

B.	Tolerances and Criteria for Acceptance

1.	Minimum grout take of _____ cubic ft. per lineal foot, with a minimum depth of _____ feet.

2.	Install piles as close as practical to design location.  Do not exceed _____ inches lateral deviation

from center of pile design location.

3.	Piles improperly installed because of mislocation, misalignment, or failure to meet other specified

design/installation criteria are not acceptable.  Abandon rejected piles and install additional piles

as required.

C.	Pile Installer Records - Maintain daily record (use Ideal Pile Installation Record) of all data pertinent

to installation of piles, including the following:

1.	Pile number

2.	Date of installation

3.	Soil Displacement Head diameter

4.	Pile shaft size

5.	Pile length

6.	Torque readings during installation

7.	Grout take per foot during installation

8.	Description of any unusual occurrences during pile construction

D.	Load Tests - Requirements

1.	Provide _____ compression/tension pile load test(s) by Pile Installer in accordance with ASTM Load Test Specifications.

2.	Load test piles incrementally at _____ times design load with deflection measured at each addition

of load.

3.	Submit detail drawings, design data and installation procedures pertaining to proposed pile load

by Pile Installer for approval 14 days prior to initiation of pile installation.

4.	Furnish and install complete load test system including jacks, reaction beam, dial indicators, spherical

bearing plate, load cell, reference beams, test enclosure, and all other equipment, materials and labor

that will satisfactorily perform required pile load test.

5.	Test piles, if successfully tested and properly located, will be accepted as permanent and may be

left in place

E.	Quality Control/Inspection:

1.	Owner's Geotechnical Engineer is to observe all pile installations and load tests.

____________________________________________________________________________________________

End of Section